Platform Baseline Standard

SEAS + Sunthetics Edge Architecture

Grid-Integrated CHP2X & Energy Banking Systems

1. Scope & Architectural Context

This standard governs all computing nodes within the Sunthetics CHP2X ecosystem, including:

- SEAS Edge Servers

- SCWE/Methanol reactor controllers

- CHP generator control nodes

- DAC control nodes

- Local energy storage control modules

- Grid/EMS interface gateways

- Energy data attestation nodes (Proof-of-Energy)

The architecture consists of an OT Layer (Unipi PLCs), an Edge Layer (SEAS industrial Linux servers), and a Cloud/Distributed Layer for analytics, redundancy, and verification.

2. Operating System Standard (SEAS Edge Nodes)

2.1 Approved OS Profile

- Industrial Linux LTS (Ubuntu LTS or Debian Stable preferred)

- Kernel LTS with optional PREEMPT_RT if required

- Minimum 5-year security maintenance window

2.2 Role Separation

Each SEAS edge node shall logically separate control services, data acquisition, optimization/AI modules, grid/EMS interfaces, blockchain/attestation modules, and system management services via container isolation.

3. OT/PLC Integration Layer (Unipi + SEAS)

3.1 Deterministic Control Boundary

All hard real-time safety logic remains in PLC, including pressure control, thermal protection, hydrogen monitoring, DAC airflow control, and emergency shutdown.

3.2 Communication Interface

- Modbus TCP or OPC UA (preferred)

- Read-only supervisory access for AI modules

- Restricted write access with strict validation layer

4. Container Runtime & Service Architecture

All SEAS services must run in immutable, version-controlled containers.

Standard services include data collection, telemetry normalization, grid connectors, optimization engine, forecast engine, event management, Proof-of-Energy module, OTA agent, and monitoring agent.

5. Local Autonomy & Fail-Safe Behavior

System must maintain safe operation during communication loss and buffer telemetry locally.

PLC executes shutdown independently during critical faults such as pressure anomalies or hydrogen leaks.

6. Secure Communication Architecture

- Unique device identity per node

- Certificate-based authentication (mTLS)

- Network segmentation between OT, internal services, grid interface, and external cloud

7. Observability & Energy Traceability

Required telemetry includes power, methanol production, hydrogen generation, pressure, temperature, DAC rate, storage state, and efficiency KPIs.

All telemetry must be time-synchronized and cryptographically protected.

8. AI / Optimization Modules

AI modules operate in sandboxed containers with read-only telemetry access.

Safety-critical parameters remain enforced in PLC and cannot be overridden by AI.

9. Over-the-Air (OTA) & Lifecycle Management

Updates must be digitally signed with rollback capability and staged deployment support.

Safety-critical PLC updates require controlled downtime procedures.

10. Distributed Data Center Compatibility

SEAS nodes must support horizontal replication, distributed storage sync, and redundant energy attestation.

Edge systems must remain operational if isolated from the distributed cluster.

11. Reference SEAS + Sunthetics Node Profile

Each compliant node includes hardened Linux LTS, container runtime, PLC drivers, grid adapter, local AI optimizer, monitoring stack, OTA agent, cryptographic module, secure storage, and watchdog.

12. Strategic Design Principle

The SEAS + Sunthetics platform is built on physical autonomy, deterministic safety, cryptographic energy integrity, modular scalability, and grid interoperability without grid dependency.

The digital layer must never introduce instability into the physical energy layer.